微算機系統

小組專案報告

期末專案：

CPU Pipeline實現

組別： 20

班級、姓名與學號：

醫工三 葉芸茜 B812110004

醫工三 湯青秀 B812110011

日期 :112年1月11日

一、實驗內容：

- 所有功能皆須以pipeline實現才有分數

- 總共有四個暫存器(R0,R1,R2,R3)，使用RS和RT分別指定要使用的暫存器(七段顯示器須同步更新)

- 使用七段顯示器顯示 Rs、Rt 以及 Data 的值

- Load/Move 是必要完成的功能，需實現所有功能本次實驗才有分數

- 七段顯示器顯示為十六進制，兩顆七段顯示器為一組，範圍為1~FF (超過FF顯示末兩位)

- 左邊兩組顯示 Rs 及 Rt 兩個暫存器(EX stage階段的Rs & Rt值 )

- 執行指令時，若暫存器的內容有被改變，則上述兩組七段顯示器須同步更新

- 七段顯示器的顯示範圍為 0-FF

- 在執行各 Cycle 時，須以綠色 LED 燈顯示該 Cycle 是否正在執行

- 偵測 Data Hazard時，須以紅色 LED 燈顯示是否有 Hazard 產生

| Variable | Pin Location | Signal Name |
| --- | --- | --- |
| clk | PIN_M23 | KEY[0] |
| output[0] | PIN_F19 | LED[1] |
| output[1] | PIN_E19 | LED[2] |
| output[2] | PIN_F21 | LED[3] |
| data(0) | PIN_AB28 | SW[0] |
| data(1) | PIN_AC28 | SW[1] |
| data(2) | PIN_AC27 | SW[2] |
| data(3) | PIN_AD27 | SW[3] |
| data(4) | PIN_AB27 | SW[4] |
| data(5) | PIN_AC26 | SW[5] |
| data(6) | PIN_AD26 | SW[6] |
| data(7) | PIN_AB26 | SW[7] |
| opcode(0) | PIN_AC25 | SW[8] |
| opcode(1) | PIN_AB25 | SW[9] |
| opcode(2) | PIN_AC24 | SW[10] |
| opcode(3) | PIN_AB24 | SW[11] |
| rs(0) | PIN_AB23 | SW[12] |
| rs(1) | PIN_AA24 | SW[13] |
| rt(0) | PIN_AA23 | SW[14] |
| rt(1) | PIN_AA22 | SW[15] |
| a0 | PIN_G18 | HEX0[0] |
| b0 | PIN_F22 | HEX0[1] |
| c0 | PIN_E17 | HEX0[2] |
| d0 | PIN_L26 | HEX0[3] |
| e0 | PIN_L25 | HEX0[4] |
| f0 | PIN_J22 | HEX0[5] |
| g0 | PIN_H22 | HEX0[6] |
| a1 | PIN_M24 | HEX1[0] |
| b1 | PIN_Y22 | HEX1[1] |
| c1 | PIN_W21 | HEX1[2] |
| d1 | PIN_W22 | HEX1[3] |
| e1 | PIN_W25 | HEX1[4] |
| f1 | PIN_U23 | HEX1[5] |
| g1 | PIN_U24 | HEX1[6] |
| a2 | PIN_AA25 | HEX2[0] |
| b2 | PIN_AA26 | HEX2[1] |
| c2 | PIN_Y25 | HEX2[2] |
| d2 | PIN_W26 | HEX2[3] |
| e2 | PIN_Y26 | HEX2[4] |
| f2 | PIN_W27 | HEX2[5] |
| g2 | PIN_W28 | HEX2[6] |
| a3 | PIN_V21 | HEX3[0] |
| b3 | PIN_U21 | HEX3[1] |
| c3 | PIN_AB20 | HEX3[2] |
| d3 | PIN_AA21 | HEX3[3] |
| e3 | PIN_AD24 | HEX3[4] |
| f3 | PIN_AF23 | HEX3[5] |
| g3 | PIN_Y19 | HEX3[6] |
| a4 | PIN_V21 | HEX4[0] |
| b4 | PIN_U21 | HEX4[1] |
| c4 | PIN_AB20 | HEX4[2] |
| d4 | PIN_AA21 | HEX4[3] |
| e4 | PIN_AD24 | HEX4[4] |
| f4 | PIN_AF23 | HEX4[5] |
| g4 | PIN_Y19 | HEX4[6] |
| a5 | PIN_V21 | HEX5[0] |
| b5 | PIN_U21 | HEX5[1] |
| c5 | PIN_AB20 | HEX5[2] |
| d5 | PIN_AA21 | HEX5[3] |
| e5 | PIN_AD24 | HEX5[4] |
| f5 | PIN_AF23 | HEX5[5] |
| g5 | PIN_Y19 | HEX5[6] |
| h1 (hazard LED1) | PIN_H21 | LEDG[4] |
| h2 (hazard LED2) | PIN_G20 | LEDG[5] |
| IF_LED | PIN_E21 | LEDG[0] |
| ID_LED | PIN_E22 | LEDG[1] |
| EX_LED | PIN_E25 | LEDG[2] |
| WB_LED | PIN_E24 | LEDG[3] |
| mux1_choose(0) | PIN_J17 | LED[8] |
| mux1_choose(1) | PIN_G17 | LED[9] |
| mux2_choose(0) | PIN_J15 | LED[10] |
| mux2_choose(1) | PIN_H16 | LED[11] |
| rs_choose(0) | PIN_J16 | LED[12] |
| rs_choose(1) | PIN_H17 | LED[13] |
| rt_choose(0) | PIN_F15 | LED[14] |
| rt_choose(1) | PIN_G15 | LED[15] |
| result(0) | PIN_G19 | LED[0] |
| result(1) | PIN_F19 | LED[1] |
| result(2) | PIN_E19 | LED[2] |
| result(3) | PIN_F21 | LED[3] |
| result(4) | PIN_F18 | LED[4] |
| result(5) | PIN_E18 | LED[5] |
| result(6) | PIN_J19 | LED[6] |
| result(7) | PIN_H19 | LED[7] |

二、實驗過程及結果：

（一） 預期實驗結果的流程示意圖

（二） 設計電路簡介

- 程式程序

  - IFetch(process):

當該次輸入為nop則關閉LED，否則偵測前一次rs是否與該次rs rt有相同，再將rs, rt. data傳入IF暫存器，LED燈亮。

將opcode傳入IF暫存器。

  - IDecode(process):

先偵測輸入rs rt是否與前兩次的rs相同。

再看IF暫存器中opcode是否為nop，是則關閉LED，否則LED亮，並依opcode將需要用的暫存器值(在r0, r1, r2, r3中)存入ID暫存器。

將rs, rt. opcode傳入ID暫存器。

  - EXE(process):

看ID暫存器中opcode是否為nop，是則關閉LED，否則LED亮，並依opcode將適當的值做為result存入EX暫存器。

將rs, rt. opcode傳入EX暫存器。

  - WBack(process):

看EX暫存器中opcode是否為nop，是則關閉LED，否則LED亮，EX中result, rs, rt放入WB，並依EX中的rs選擇將result放回該暫存器。

  - Hazard:

根據偵測得到該條指令mux為00,01或10，對IF ID兩邊偵測得到是否有hazard發生的值(0或1)做OR得到LED控制數。

依mux來選取要傳進EXE做運算的s_temp, t_temp和ID_move。

- 最終結果

具有基本計算功能(除div)，且具有對write back stage的forwarding(與hazard之程序須間隔 1 nop)。

（三） 設計出的電路

（四） 實驗過程

- 撰寫程式碼

- 編譯成功

- 接腳位

- 確認接線於正確腳位

- 燒錄視窗設定及燒錄成功畫面

（五） 實驗結果

- 實際操作影片連結:

測資1:

https://drive.google.com/file/d/1k-cTR-kuigpimAvi5fCPxrvUd_UYQfzn/view?usp=sharing

測資2:

https://drive.google.com/file/d/19S_wXKFlPvW0hx9fp7W5lsUD5ojyO2xB/view?usp=sharing

測資3:

https://drive.google.com/file/d/10me9L5hqGVL7PSYj_FkU85_dH3ZxKeDa/view?usp=sharing

三、程式碼

pipeline.vhd

LIBRARY IEEE; USE IEEE.STD_LOGIC_1164.ALL; USE IEEE.STD_LOGIC_UNSIGNED.ALL; USE work.course8_package.all; ENTITY pipeline IS PORT( clk: IN STD_LOGIC; data: IN STD_LOGIC_VECTOR(7 DOWNTO 0); opcode: IN STD_LOGIC_VECTOR(3 downto 0); rs, rt: IN STD_LOGIC_VECTOR(1 DOWNTO 0); a0, b0, c0, d0, e0, f0, g0, a1, b1, c1, d1, e1, f1, g1, a2, b2, c2, d2, e2, f2, g2, a3, b3, c3, d3, e3, f3, g3, a4, b4, c4, d4, e4, f4, g4, a5, b5, c5, d5, e5, f5, g5: OUT std_logic; IF_LED, ID_LED, EX_LED, WB_LED: out std_logic; result: out std_logic_vector(7 downto 0); h1, h2: out std_logic; rs_choose, rt_choose, mux1_choose, mux2_choose: out STD_logic_vector(1 downto 0) ); END pipeline; architecture main of pipeline is SIGNAL IF_op, ID_op, EX_op, WB_op: std_logic_vector(3 downto 0):="1111"; SIGNAL IF_rs, IF_rt, ID_rs, ID_rt, EX_rs, EX_rt, WB_rs, WB_rt: std_logic_vector(1 downto 0); SIGNAL ID_s, ID_t, ID_lm, ID_move, EX_result, EX_old , WB_result: std_logic_vector(7 downto 0); SIGNAL IF_data: std_logic_vector(7 downto 0); SIGNAL IF1_mux1, IF1_mux2, IF_mux1, IF_mux2, ID_mux1, ID_mux2, EXMux1, EXMux2: std_logic_vector(1 downto 0):="00"; SIGNAL q, s_temp, t_temp, lm: std_logic_vector(7 downto 0); SIGNAL r0, r1, r2, r3: std_logic_vector(7 downto 0); SIGNAL show_s, show_t: std_logic_vector(7 downto 0); SIGNAL ifh1, ifh2, idh1, idh2: std_logic; begin IFetch: process begin wait until clk'event and clk = '1'; IF1_mux1 <= "00"; ifh1 <= '0'; IF1_mux2 <= "00"; ifh2 <= '0'; if opcode = "1111" then IF_LED <= '0'; else if opcode /= "0000" then if IF_op /= "1111" then if rs = IF_rs then IF1_mux1 <= "01"; ifh1 <= '1'; end if; else IF1_mux1 <= "00"; ifh1 <= '0'; end if; if IF_op /= "1111" then if rt = IF_rs then IF1_mux2 <= "01"; ifh2 <= '1'; end if; else IF1_mux2 <= "00"; ifh2 <= '0'; end if; end if; IF_rs <= rs; IF_rt <= rt; IF_data <= data; IF_LED <= '1'; end if; IF_op <= opcode; end process; IDecode: process begin wait until clk'event and clk = '1'; ID_mux1 <= "00"; idh1 <= '0'; ID_mux2 <= "00"; idh2 <= '0'; ID_mux1 <= IF_mux1; ID_mux2 <= IF_mux2; IF_mux1 <= "00"; IF_mux2 <= "00"; If IF1_mux1 = "01" then IF_mux1 <= IF1_mux1; else if opcode /= "0000" and opcode /= "1111" then if ID_op /= "1111" then if rs = ID_rs then IF_mux1 <= "10"; idh1 <= '1'; end if; else IF_mux1 <= "00"; idh1 <= '0'; end if; end if; End if; If IF1_mux2 = "01" then IF_mux2 <= IF1_mux2; else if opcode /= "0000" and opcode /= "1111" then if ID_op /= "1111" then if rt = ID_rs then IF_mux2 <= "10"; idh2 <= '1'; end if; else IF_mux2 <= "00"; idh2 <= '0'; end if; End if; end if; if IF_op = "1111" then ID_LED <= '0'; else ID_LED <= '1'; if IF_op(3 downto 1) = "000" then case IF_op(0) is when '0' => ID_lm <= IF_data; when '1' => case IF_rt is when "00" => ID_lm <= r0; when "01" => ID_lm <= r1; when "10" => ID_lm <= r2; when "11" => ID_lm <= r3; end case; end case; else case IF_rs is when "00" => ID_s <= r0; when "01" => ID_s <= r1; when "10" => ID_s <= r2; when "11" => ID_s <= r3; end case; case IF_rt is when "00" => ID_t <= r0; when "01" => ID_t <= r1; when "10" => ID_t <= r2; when "11" => ID_t <= r3; end case; end if; end if; ID_op <= IF_op; ID_rs <= IF_rs; ID_rt <= IF_rt; end process; h1 <= ifh1 or idh1; h2 <= ifh2 or idh2; divise: diviser PORT MAP(clk,'0', t_temp, s_temp, q); EXE:process begin wait until clk'event and clk = '1'; if ID_op = "1111" then EX_LED <= '0'; else EX_LED <= '1'; if ID_op= "0000" then EX_result <= ID_lm; elsif ID_op = "0001" then EX_result <= ID_move; else case ID_op is when "0010" => EX_result <= s_temp + t_temp; when "0011" => EX_result <= s_temp - t_temp; when "0100" => EX_result <= s_temp and t_temp; when "0101" => EX_result <= s_temp or t_temp; when "0110" => EX_result <= s_temp nor t_temp; when "0111" => if s_temp < t_temp then EX_result <= "00000001"; else EX_result <= "00000000"; end if; when "1000" => EX_result <= q; when others => end case; end if; end if; result <= EX_result; EX_old <= EX_result; EX_op <= ID_op; EX_rs <= ID_rs; EX_rt <= ID_rt; end process; EXMux1 <= ID_mux1; EXMux2 <= ID_mux2; with EXMux1 select s_temp <= ID_s when "00", EX_old when "01", WB_result when "10", ID_s when others; with EXMux2 select t_temp <= ID_t when "00", EX_old when "01", WB_result when "10", ID_t when others; with EXMux2 select ID_move <= ID_lm when "00", EX_old when "01", WB_result when "10", ID_lm when others; WBack:process begin wait until clk'event and clk = '1'; if EX_op = "1111" then WB_LED <= '0'; else WB_result <= EX_result; WB_rs <= EX_rs; WB_rt <= EX_rt; WB_LED <= '1'; case EX_rs is when "00" => r0 <= EX_result; when "01" => r1 <= EX_result; when "10" => r2 <= EX_result; when "11" => r3 <= EX_result; end case; end if; end process; show_select: with WB_rs select show_s <= r0 when "00", r1 when "01", r2 when "10", r3 when "11"; with WB_rt select show_t <= r0 when "00", r1 when "01", r2 when "10", r3 when "11"; rs_choose <= WB_rs; rt_choose <= WB_rt; mux1_choose <= EXMux1; mux2_choose <= ExMux2; show_rs: seven_segment PORT MAP(show_s(3), show_s(2), show_s(1), show_s(0), show_s(7), show_s(6), show_s(5), show_s(4), a1, b1, c1, d1, e1, f1, g1, a0, b0, c0, d0, e0, f0, g0); show_rt: seven_segment PORT MAP(show_t(3), show_t(2), show_t(1), show_t(0), show_t(7), show_t(6), show_t(5), show_t(4), a3, b3, c3, d3, e3, f3, g3, a2, b2, c2, d2, e2, f2, g2); show_bus: seven_segment PORT MAP(data(3), data(2), data(1), data(0), data(7), data(6), data(5), data(4), a4, b4, c4, d4, e4, f4, g4, a5, b5, c5, d5, e5, f5, g5); end main;

package.vhd

LIBRARY ieee; USE ieee.std_logic_1164.all; PACKAGE course8_package IS component seven_segment PORT( W0, X0, Y0, Z0, W1, X1, Y1, Z1: IN STD_LOGIC; a0, b0, c0, d0, e0, f0, g0, a1, b1, c1, d1, e1, f1, g1: OUT STD_LOGIC); END component seven_segment; component diviser PORT( clk, clear: IN STD_LOGIC; divisor, dividend: IN STD_LOGIC_VECTOR(7 downto 0); q : out STD_LOGIC_VECTOR(7 downto 0) ); END component diviser; END course8_package;

seven_segment.vhd

Library ieee; USE ieee.std_logic_1164.all; ENTITY seven_segment IS PORT( W0, X0, Y0, Z0, W1, X1, Y1, Z1 : IN STD_LOGIC; a0, b0, c0, d0, e0, f0, g0, a1, b1, c1, d1, e1, f1, g1: OUT STD_LOGIC); END seven_segment; ARCHITECTURE LogicFunc OF seven_segment IS BEGIN a0 <= (NOT W0 AND NOT X0 AND NOT Y0 AND Z0) OR (NOT W0 AND X0 AND NOT Y0 AND NOT Z0) OR (W0 AND NOT X0 AND Y0 AND Z0) OR (W0 AND X0 AND NOT Y0); b0 <= (NOT W0 AND X0 AND NOT Y0 AND Z0) OR (NOT W0 AND X0 AND Y0 AND NOT Z0) OR (W0 AND NOT X0 AND Y0 AND Z0) OR (W0 AND X0 AND NOT Z0) OR (W0 AND X0 AND Y0); c0 <= (NOT W0 AND NOT X0 AND Y0 AND NOT Z0) OR (W0 AND X0 AND NOT Z0) OR (W0 AND X0 AND Y0); d0 <= (NOT X0 AND NOT Y0 AND Z0) OR (NOT W0 AND X0 AND NOT Y0 AND NOT Z0) OR (X0 AND Y0 AND Z0) OR (W0 AND NOT X0 AND Y0 AND NOT Z0); e0 <= (NOT W0 AND Z0) OR (NOT W0 AND X0 AND NOT Y0) OR (NOT X0 AND NOT Y0 AND Z0); f0 <= (NOT W0 AND NOT X0 AND Z0) OR (NOT W0 AND NOT X0 AND Y0) OR (NOT W0 AND Y0 AND Z0) OR (W0 AND X0 AND NOT Y0); g0 <= (NOT W0 AND NOT X0 AND NOT Y0) OR (NOT W0 AND X0 AND Y0 AND Z0); a1 <= (NOT W1 AND NOT X1 AND NOT Y1 AND Z1) OR (NOT W1 AND X1 AND NOT Y1 AND NOT Z1) OR (W1 AND NOT X1 AND Y1 AND Z1) OR (W1 AND X1 AND NOT Y1); b1 <= (NOT W1 AND X1 AND NOT Y1 AND Z1) OR (NOT W1 AND X1 AND Y1 AND NOT Z1) OR (W1 AND NOT X1 AND Y1 AND Z1) OR (W1 AND X1 AND NOT Z1) OR (W1 AND X1 AND Y1); c1 <= (NOT W1 AND NOT X1 AND Y1 AND NOT Z1) OR (W1 AND X1 AND NOT Z1) OR (W1 AND X1 AND Y1); d1 <= (NOT X1 AND NOT Y1 AND Z1) OR (NOT W1 AND X1 AND NOT Y1 AND NOT Z1) OR (X1 AND Y1 AND Z1) OR (W1 AND NOT X1 AND Y1 AND NOT Z1); e1 <= (NOT W1 AND Z1) OR (NOT W1 AND X1 AND NOT Y1) OR (NOT X1 AND NOT Y1 AND Z1); f1 <= (NOT W1 AND NOT X1 AND Z1) OR (NOT W1 AND NOT X1 AND Y1) OR (NOT W1 AND Y1 AND Z1) OR (W1 AND X1 AND NOT Y1); g1 <= (NOT W1 AND NOT X1 AND NOT Y1) OR (NOT W1 AND X1 AND Y1 AND Z1); END LogicFunc;

diviser.vhd

LIBRARY IEEE; USE IEEE.STD_LOGIC_1164.ALL; USE IEEE.STD_LOGIC_UNSIGNED.ALL; ENTITY diviser IS PORT( clk,clear: IN STD_LOGIC; divisor, dividend: IN STD_LOGIC_VECTOR(7 downto 0); q : out STD_LOGIC_VECTOR(7 downto 0) ); END diviser; architecture main of diviser is signal w : STD_LOGIC :='0'; signal re : INTEGER:= 0; signal run : std_logic; signal temp_s, temp_p: STD_LOGIC_VECTOR(15 downto 0); signal output : STD_LOGIC_VEcTOR(2 downto 0); signal remainder: STD_LOGIC_VECTOR(15 DOWNTO 0); begin PROCESS begin WAIT UNTIL Clk'EVENT AND clk = '1'; if clear = '1' then output <= "100"; else Case output is when "000" => output <= "111"; when "111" => output <= "001"; when "001" => if w = '1' then output <= "011"; else output <= "010"; end if; when "011" => if w = '1' then output <= "101"; else output <= "001"; end if; when "010" => if w = '1' then output <= "101"; else output <= "001"; end if; when "100"=> output <= "000"; when others => output <= "110"; end Case; end if; end PROCESS; case3: temp_s <= (( remainder(15 downto 8) - divisor) & remainder(7 downto 0)); case4: temp_p <= (( remainder(15 downto 8) + divisor) & remainder(7 downto 0)); output_select: process begin WAIT UNTIL Clk'EVENT AND clk = '1'; case output is when "000" => remainder <= ("00000000" & dividend); when "111" => for_1: FOR i IN 14 downto 0 loop remainder(i + 1) <= remainder(i); END loop; remainder(0) <= '0'; when "001" => remainder <= (( remainder(15 downto 8) - divisor) & remainder(7 downto 0)); when "011" => for_2: FOR i IN 14 downto 0 loop remainder(i + 1) <= remainder(i); END loop; remainder(0) <= '1'; when "010" => for_3: FOR i IN 14 downto 0 loop remainder(i + 1) <= temp_p(i); END loop; remainder(0) <= '0'; when "100"=> remainder <= "0000000000000000"; when "101" => for_r: FOR i IN 8 to 14 loop remainder(i) <= remainder(i + 1); END loop; remainder(15) <= '0'; remainder(7 downto 0) <= remainder(7 downto 0); when others => remainder <= remainder; end Case; end process; count:process begin WAIT UNTIL Clk'EVENT AND clk = '1'; if clear = '1' then re <= 0; elsif output = "001" and run = '1' then re <= re + 1; else re <= re; end if; end process; run <= '0' when re = 8 else '1'; with output select w <= not temp_s(15) when "001", not run when "011", not run when "010", '0' when others; q <= remainder(7 downto 0); end main;